# Torrio

# Factory Command Firmware Spec

Factory Command Firmware Spec

Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.01 | 2025/10/16 | Timmy Tien | First draft of firmware spec. |

Contents

1.	Introduction	4

2.	Reference Documents	5

3.	USB Description	6

3.1.	Torrio	6

3.1.1.	Case	6

3.1.2.	Dongle	6

3.2.	Interfaces & Endpoints	7

3.2.1.	Case	7

3.2.2.	Dongle	8

3.3.	Command Format	8

3.3.1.	Set_Report Format	8

3.3.2.	Get_Report Format	9

3.4.	File Commands	9

3.5.	File Commands	9

3.5.1.	File Systems and IDs	10

4.	Command Endpoint USB HID Commands	11

4.1.	Status Commands	11

4.1.1.	Reset MCU (0x01)	11

4.1.2.	Get Firmware Version (0x90)	11

4.1.3.	Get Serial Number (0x93)	12

4.1.4.	Set Serial Number (0x13)	12

4.1.5.	Set spin color and model (0x6D)	13

4.1.6.	get spin color and model (0xED)	13

5.	HID Command for Factory Command	14

5.1.1.	Enter Factory mode (0xFC, 0xFC, 0xAA, 0x55)	14

5.1.2.	Sy8809 xSense Read (0xF0) (Csae Only)	14

5.1.3.	Sy8809 register read (0xF1) (Csae Only)	15

5.1.4.	Sy8809 reg write(0x71) (Csae Only)	15

# Introduction

The purpose of this document is to define the USB interface for Torrio. Torrio is a pair of true wireless earphones designed specifically for gaming. Torrio uses a true gaming wireless 2.4 GHz wireless radio with ultra-low latency and lossless audio. Torrio also supports Bluetooth to connect to any Bluetooth enabled host device. A compact USB-C dongle is used to provide the end-to-end wireless connectivity, providing compatibility with PC, PlayStation, Android, and Switch.

Additionally:

- In this document, TBD is a placeholder for items that need to be further defined.

- For simplicity, we simply refer to the dongle and case as the devices and the USB host as the PC.

- Each command will indicate the USB Endpoint to be used and the direction of communications, such as:

Command Endpoint: Set_Report (PC -> Device)

- This is a living document. Any requirement can be changed, modified, added, and/or deleted.

- It is up to the developer to specify any other commands needed for production and testing.

# Reference Documents

| Document Name | Date | Location |
| --- | --- | --- |
| Arctis 7+, 7P+, 7X+ Firmware Spec 1.7 | 23 Mar 2022 | https://steelseries.box.com/s/wpx9p2blk3fwrtq1w9w7asxvmvabmnj8 |
| Bumblebee and Optimus USB Firmware Spec v1.30 | 15 Apr 2022 | https://steelseries.box.com/s/h0d7kbi35tepsxp5qcs8do2dk1rppzjk |

# USB Description

## Torrio

### Case

#### Normal Mode

| Vendor ID | 0x1038 |
| --- | --- |
| Product ID | 0x2267 |
| Mfg. Name | SteelSeries |
| Product Name | Torrio Case |

#### Bootloader Mode

| Vendor ID | 0x1038 |
| --- | --- |
| Product ID | 0x2268 |
| Mfg. Name | SteelSeries |
| Product Name | Bootloader |

### Dongle

#### Normal Mode (pc)

| Vendor ID | 0x1038 |
| --- | --- |
| Product ID | 0x2269 |
| Mfg. Name | SteelSeries |
| Product Name | Torrio Dongle |

#### Bootloader Mode

| Vendor ID | 0x1038 |
| --- | --- |
| Product ID | 0x226A |
| Mfg. Name | SteelSeries |
| Product Name | SteelSeries Bootloader |

## Interfaces & Endpoints

Regarding this case, there are two parts with USB, one is the dongle and the other is the charging case. They have different listing contents., The device will have the following interfaces and endpoints.

### Case

Interfaces and endpoints.

| Interface | Endpoint | Endpoint Name | Description |
| --- | --- | --- | --- |
| 0 | 1 | HID-Compliant Vendor-Defined (Command Endpoint) | The Command Endpoint will be used by the PC to send messages to the device and for the device to respond when appropriate. In this document, these exchanges will be shown as: Command Endpoint: Set_Report (PC -> Device) and Command Endpoint: Interrupt IN (PC <- Device) |
| 1 | 2 | HID-Compliant Consumer Control | This is required so Windows does not put the device into suspend mode from not being used. |
| 2 | 3 | HID-Compliant Vendor-Defined (Sync Endpoint) | The Sync Endpoint will be used by the device to Inform the PC when a status change has occurred. In this document, these status updates will be shown as: Sync Endpoint: (PC <- Device) |

### Dongle

Interfaces and endpoints.

| Interface | Endpoint | Endpoint Name | Description |
| --- | --- | --- | --- |
| 0 | 0 | Control Transfer | Default Endpoint. |
| 3 | 1 | HID-Compliant Vendor-Defined (Command Endpoint) | The Command Endpoint will be used by the PC to send messages to the device and for the device to respond when appropriate. In this document, these exchanges will be shown as: Command Endpoint: Set_Report (PC -> Device) and Command Endpoint: Interrupt IN (PC <- Device) |
| 4 | 2 | HID-Compliant Consumer Control | This is required so Windows does not put the device into suspend mode from not being used. |
| 1 | 3 | Isochronous Transfers (Stereo Output) | UAC 1.0. Must support 16-bit 48KHz stereo audio, 24-bit support preferred. |
| 2 | 3 | Isochronous Transfers (Mic Input) | UAC 1.0. Must support a 16-bit 48KHz microphone input. |
| 5 | 4 | HID-Compliant Vendor-Defined (Sync Endpoint) | The Sync Endpoint will be used by the device to Inform the PC when a status change has occurred. In this document, these status updates will be shown as: Sync Endpoint: (PC <- Device) |

## Command Format

### Set_Report Format

Set_Report is used by the PC to send messages to the device over the Command Endpoint.

Shown below is the format of the Set_Report command. This follows the USB HID Class Standard.

| Byte 0 | Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x09 | 0x00 | REPORT TYPE | INTERFACE # | 0x00 | 0xNN | 0xMM |

| Field | Description |
| --- | --- |
| Report Type | 0x01: Input Report |
|  | 0x02: Output Report |
|  | 0x03: Feature Report |
| Interface # | The USB Command interface number |
| 0xMMNN | length of the data |

### Get_Report Format

Get_Report USB Request is only used for reading data back from device when using Feature Reports. Typically Feature Reports are only used for transferring more than 64 bytes at a time. The maximum Feature Report size needs to be at least the size of the longest message in this document. When the host wants to read data back from the device, host should issue a Set_Report first, and then issue a Get_Report to read the actual data back.

Shown below is the format of the Get_Report command. This follows the USB HID Class Standard.

| Byte 0 | Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA1 | 0x01 | 0x00 | REPORT TYPE | INTERFACE # | 0x00 | 0xNN | 0xMM |

| Field | Description |
| --- | --- |
| Report Type | 0x01: Input Report |
|  | 0x02: Output Report |
|  | 0x03: Feature Report |
| Interface # | The USB Command Interface number |
| 0xMMNN | length of the data |

## File Commands

In the following sections all commands start with a single byte op-code. The most significant bit of the op-code is used to determine if the command is a read or a write.

## File Commands

Some commands may fail. Here is the tentative list of errors for working with file commands

| ERROR CODE | Meaning |
| --- | --- |
| 0x00 | Success |
| 0x01 | File not found |
| 0x02 | File too short. I.E. targeted offset is past the file end |
| 0x03 | Flash failed. When internal flash returns error |
| 0x04 | Permission denied. USB API doesn't have permission to access this file |
| 0x05 | Operation not supported |

| Field |  | Format | Description |
| --- | --- | --- | --- |
| File System |  | uint8 | The ID of the file system targeted. When device has multiple flash devices they should be enumerated with different IDs |
| ID |  | uint8 | The ID of the file. See device details for IDs and files stored in flash |

### File Systems and IDs

| File System | Meaning |
| --- | --- |
| 0x00 | Case |
| 0x01 | Left Bud |
| 0x02 | Right Bud |

| File ID | Meaning |
| --- | --- |
| 0x00 | Application Update File ID |
| 0x01 | TBD |
| 0x02 | TBD |
| 0x03 | TBD |

# Command Endpoint USB HID Commands

This section describes the communication between a USB host and the SteelSeries devices. To write data to the device, the USB host sends the Set_Report on the Command Endpoint, in some cases defined below the device will respond back using the Interrupt IN pipe.  Additional factory, test, and programming commands may also be incorporated and sent over this Command Endpoint.   The Firmware Update Commands are sent from the Bootloader, all other commands are sent through the application.

## Status Commands

### Reset MCU (0x01)

Reset and Enter Bootloader Mode before updating the firmware.

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Byte 1 | Byte 2 |
| --- | --- | --- |
| 0x01 | Mode | File System |

| Value | Description |
| --- | --- |
| Mode | 0x00 - Reset and return to normal operating mode. 0x01 – Reset and stay in Bootloader mode |

### Get Firmware Version (0x90)

Retrieve the firmware versions of the MCU and Earbuds MCU.

Command Endpoint: Set_Report (PC -> Device)

Byte 0

0x90

Command Endpoint: Interrupt IN (PC <- Device)

| Byte 0 | Bytes 1-12 | Bytes 13-24 | Bytes 25-36 |
| --- | --- | --- | --- |
| 0x90 | Case MCU Version (AT32F415) | Left Bud MCU Version (RTL8763 EWM) | Right Bud MCU Version (RTL8763 EWM) |

If the headset is not connected to the dongle, the dongle will return all 0x00 for the headset.

This is ASCII

| Firmware Version | Value of ASCII |
| --- | --- |
| 00.00.01 | 0x30, 0x30, 0x2E, 0x30, 0x30, 0x2E, 0x30, 0x31, 0x00, 0x00, 0x00, 0x00 |
| 01.09.02 | 0x30, 0x31, 0x2E, 0x30, 0x39, 0x2E, 0x30, 0x32, 0x00, 0x00, 0x00, 0x00 |

### Get Serial Number (0x93)

This command will read the 19-byte serial number.

Command Endpoint: Set_Report (PC -> Device)

Byte 0

0x93

Command Endpoint: Interrupt IN (PC <- Device)

| Byte 0 | Bytes 1-19 | Bytes 20-38 | Bytes 39-57 |
| --- | --- | --- | --- |
| 0x93 | Case Serial Number | Left Bud Serial Number | Right Bud Serial Number |

Note: If the dongle is not connected to the headset, the headset serial number will return all 0x00.

### Set Serial Number (0x13)

This command will write the 19-byte serial number. The serial number can only be changed by this command. No other commands can change the serial number. The write operation is protected by a security key which is verified by the device. This command should only be used by the factory.

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Bytes 1 | Bytes 2-5 | Bytes 6-24 |
| --- | --- | --- | --- |
| 0x13 | File System | Key | Serial Number |

Byte 0

0x13

Command Endpoint: Set_Report (PC <- Device)

### Set spin color and model (0x6D)

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Bytes 1 | Bytes 2 | Bytes 3 |
| --- | --- | --- | --- |
| 0x6D | File System | model | Color |

| Value | Model |
| --- | --- |
| 0x00 | PC/PS |
| 0x01 | XBOX |
| 0xFF | Unknown |

| Value | Color |
| --- | --- |
| 0x00 | Black |
| 0x01 | White |
| 0x02 | Glorange |
| 0xFF | Unknown |

| Byte 0 | Byte 1 |
| --- | --- |
| 0x6D | status |

Command Endpoint: Set_Report (PC <- Device)

### get spin color and model (0xED)

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Bytes 1 |
| --- | --- |
| 0xED | File System |

Command Endpoint: Set_Report (PC <- Device)

| Byte 0 | Bytes 1 | Bytes 2 |
| --- | --- | --- |
| 0xED | model | Color |

### Get Battery Status (0xB7)

The current battery level and charging status for the battery being used in the earbuds and the battery in the

Charger.

Command Endpoint: Set_Report (PC -> Device)

Byte 0

0xB7

Command Endpoint: Set_Report (PC <- Device)

| Byte 0 | Bytes 1 | Bytes 2 | Bytes 3 | Bytes 4 | Bytes 5 | Bytes 6 |
| --- | --- | --- | --- | --- | --- | --- |
| 0xB7 | Case battery level | Case Battery Charging status | Left battery level | Left Battery Charging status | Right battery level | Right Battery Charging status |

| buds/Case Battery Level | Level |
| --- | --- |
| 0x00 – 0x64 | 0 – 100% |

| Status | Battery Charging Status |
| --- | --- |
| 0x00 | UNPLUGGED/DISCHARGING - Sent when the earbud is unplugged |
| 0x01 | PLUGGED IN & CHARGING - Sent when the earbud is plugged in, and the battery is < 100% |
| 0x02 | PLUGGED IN & NOT CHARGING - Sent when the earbud is plugged in, and the battery is already at 100% OR when it reaches 100% |

# HID Command for Factory Command

### Enter Factory mode (0xFC, 0xFC, 0xAA, 0x55)

This command will entry factory mode. Will be able to verify the hardware. If wants to EXIT, just need to re-plug or send “Reset Device command”.

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Byte 1 | Byte 2 | Byte 3 | Byte 4 |
| --- | --- | --- | --- | --- |
| 0xFC | 0xFC | 0xAA | 0x55 | File System |

Command Endpoint: Interrupt IN (PC <- Device)

Byte 0

0xFC

### Sy8809 xSense Read (0xF0) (Csae Only)

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Byte 1 |
| --- | --- |
| 0xF0 | Read xSense Target |

| Value | Read xSense Target |
| --- | --- |
| 0x01 | NTC |
| 0x02 | IBAT |
| 0x03 | IVOR |
| 0x04 | IVOL |
| 0x05 | IVIN |
| 0x06 | VBAT |
| 0x07 | VBIN |

Command Endpoint: Interrupt IN (PC <- Device)

| Byte 0 | Byte 1 | Byte 2 | Byte 3 | Byte 4 |
| --- | --- | --- | --- | --- |
| 0xF0 | Adc_raw_L | Adc_raw_H | Voltage_L | Voltage_H |

Ex:

Return the data is 0xF0, 0x3B, 0x05, 0x37, 0x04

0x3B, 0x05 means ADC Raw data is 1339.

0x37, 0x04 means voltage is 1.079 V.

### Sy8809 register read (0xF1) (Csae Only)

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Byte 1 |
| --- | --- |
| 0xF1 | register |

register

Please refer to the registers in the Sy8809 datasheet

Command Endpoint: Interrupt IN (PC <- Device)

Byte 0

Receive data

### Sy8809 reg write(0x71) (Csae Only)

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Byte 1 | Byte 2 |
| --- | --- | --- |
| 0x71 | register | data |

Register/data

Please refer to the registers in the Sy8809 datasheet

Command Endpoint: Interrupt IN (PC <- Device)

| Byte 0 | Byte 1 |
| --- | --- |
| 0x71 | status |

### Read battery and NTC (0xF2) (Csae Only)

The command will get all battery and NTC status.

Command Endpoint: Set_Report (PC -> Device)

Byte 0

0xF2

Command Endpoint: Interrupt IN (PC <- Device)

| Byte 0 | Byte 1-2 | Byte 3-4 | Byte 5-6 | Byte 7-8 | Byte 9-10 | Byte 11-12 |
| --- | --- | --- | --- | --- | --- | --- |
| 0xF2 | Case_Battery | Case_NTC | Left_Battery | Left_NTC | Right_Battery | Right_NTC |

### Set LED On/Off (0x73)

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Byte 1 | Byte 2 | Byte 3 | Byte 4 |
| --- | --- | --- | --- | --- |
| 0x73 | Component | R | G | B |

| Value | Component |
| --- | --- |
| 0x00 | AT32F415 |
| 0x01 | Left Earbud |
| 0x02 | Right Earbud |

| Value | LED_State |
| --- | --- |
| 0x00 | OFF |
| 0x01 | ON |

Command Endpoint: Interrupt IN (PC <- Device)

Byte 0

0x73

### Set Start/Stop Charging (0x74) (Csae Only)

The command will set start or stop charging.

Command Endpoint: Set_Report (PC -> Device)

| Byte 0 | Byte 1 | Byte 2 |
| --- | --- | --- |
| 0x74 | Target | State |

| Value | Target |
| --- | --- |
| 0x00 | Case |
| 0x01 | Left Bud |
| 0x02 | Right Bud |

| Value | State |
| --- | --- |
| 0x00 | Stop Charging |
| 0x01 | Start Charging |

Command Endpoint: Interrupt IN (PC <- Device)

Byte 0

0x74

# END OF DOCUMENT